| SRS_139_180 | The STC Program shall monitor the WOW Data ARINC messages from DMCA for correct update and valid data according to the following logic:- IF( WOW Data ARINC message received) THEN IF ( Nose_WOW_Valid ) AND ( ARINC SSM = Normal Operation ) AND ( SDI = DMC Ch-A ) AND ( Parity is ODD ) THEN NLG_WOW1 = Nose WOW State (bit 15 of message) NLG_WOW1_Valid = TRUE ELSE NLG_WOW1_Valid = FALSE ENDIF ELSE NLG_WOW1_Valid = FALSE ENDIF Where Nose_WOW_Valid is defined as Bit 16 of ARINC Label 022 defined in Table 6.5-12 ARINC_SSM is defined as Bits 31..30 of ARINC Label 022 defined in Table 6.5-12 SDI is defined as Bits 10..9 of ARINC Label 022 defined in Table 6.5-12 Parity is ODD or EVEN defined as the sum of all the bits in ARINC Label 022 defined in Table 6.5-12 | MRJ_SCU_STC_SRS_139 |
| --- | --- | --- |
| SRS_139_180 | The STC Program shall debounce the NLG_WOW1 signal using the following algorithm to generate a filtered version of the signal IF (NLG_WOW1 has been TRUE for the last 1.5s) THEN Debounced_NLG_WOW1 = TRUE ELSE IF (NLG_WOW1 has been FALSE for the last 200ms) THEN Debounced_NLG_WOW1 = FALSE ELSE Debounced_NLG_WOW1 remains unchanged, with an initial value of FALSE ENDIF Where NLG_WOW1 is the value calculated in MRJ_SCU_STC_SRS_139 | MRJ_SCU_STC_SRS_180 |
| SRS_585 | The STC Program shall enter and remain in Failsafe Level 1 when a failure is reported by any one of the following Monitors: LVDT Double Loss Monitor defined in MRJ_SCU_STC_SRS_1245, ASA Jump Monitor Status defined in MRJ_SCU_STC_SRS_1247, LVDT No Angle Found Fault defined in MRJ_SCU_STC_SRS_1240, EHSV1 and EHSV2 Monitor defined in MRJ_SCU_STC_SRS_605, Target Input Monitor is defined in MRJ_SCU_STC_SRS_608, Over Angle Monitor defined in MRJ_SCU_STC_SRS_1212, Steering Angle Monitor defined in MRJ_SCU_STC_SRS_1214, Watchdog Test Monitor defined in MRJ_SCU_STC_SRS_308, Watchdog Maximum Reset Count Monitor defined in MRJ_SCU_STC_SRS_476, ROM Test defined in MRJ_SCU_STC_SRS_478, MRJ_SCU_STC_SRS_479 and MRJ_SCU_STC_SRS_614 RAM Integrity test defined in MRJ_SCU_STC_SRS_306 and MRJ_SCU_STC_SRS_512 Rigging record and data Check defined in MRJ_SCU_STC_SRS_1043 Note1: Each of these individual Monitors is defined as non-recoverable, therefore ensuring the STC Program cannot leave Failsafe level 1 after any one of these has been reported. Note2:The ROM Test and Ram Test are classified as a recoverable Monitors in the Supplier Equipment Specification (Ref : B2) however this is only after a reset of the Control Lane. Note3: Some of the above fault conditions result in procesor resets which eventually end up with the processor stopping entry into a run-time mode. The SCU, as a whole, then behaves as if it is in Failsafe Level 1 because, firstly, the affected processor disables the SOV and secondly, the other processor will be in Failsafe Level 1 because of Loss of Interlane Communications. | MRJ_SCU_STC_SRS_585 |
| SRS_811 | The STC Program shall determine A_DMC_9 LADC Monitor Status according to the following logic: IF the following condition is true for 320ms with a tolerance of +10ms (( No valid A_DMC_9 LADC labels have been receive ) OR (A_DMC_9 LADC data have wrong (even) parity bit) OR (A_DMC_9 LADC have SSM data not set to "Normal Operation")) THEN A_DMC_9 LADC Monitor Status = TRUE ELSE IF the following condition is true for 320ms with a tolerance of +10ms ((A_DMC_9 LADC labels have been received at expected update rate with a tolerance of +10ms ) AND (A_DMC_9 LADC data have correct (odd) parity bit) AND (A_DMC_9 LADC have SSM data set to "Normal Operation") AND (A_DMC_9 LADC Airspeed < 35kts)) THEN A_DMC_9 LADC Monitor Status = FALSE ELSE No action (A_DMC_9 LADC Monitor Status remains unchanged) ENDIF ENDIF Where A_DMC_9 LADC ARINC Message is defined in MRJ_SCU_STC_SRS_135 Justification for timing tolerance: ARINC label reception requires tolerance for label polling and processing. | MRJ_SCU_STC_SRS_811 |
| SRS_181 | The STC Program shall debounce the NLG_WOW2 signal using the following algorithm to generate a filtered version of the signal IF (NLG_WOW2 has been TRUE for the last 1.5s) THEN Debounced_NLG_WOW2 = TRUE ELSE IF (NLG_WOW2 has been FALSE for the last 200ms) THEN Debounced_NLG_WOW2 = FALSE ELSE Debounced_NLG_WOW2 remains unchanged, with an initial value of FALSE ENDIF Where NLG_WOW2 is the value calculated in MRJ_SCU_STC_SRS_142 |  |